UART PROTOCOL

- Explanation

Universal Asynchronous Receiver Transmitter (UART) is a widely used hardware communication protocol that enables asynchronous serial data transfer between digital devices. Unlike synchronous communication methods, UART does not use a shared clock signal between the transmitter and the receiver. Instead, both sides operate with a predefined baud rate, which determines the duration of each transmitted bit.

Figure 1: Physical UART connection between two devices showing TX, RX and common ground lines.

Figure 1 shows the basic physical connection of two UART-enabled devices. Each device has a transmit (TX) and a receive (RX) line. The TX pin of one device is connected to the RX pin of the other device, while both systems share a common ground (GND). This cross-connection allows full-duplex communication between the two devices.

UART communication is based on framing the data with control bits. A general UART frame structure is illustrated in Figure 2.

Figure 2: General UART frame structure including start bit, data bits, optional parity bit and stop bit(s).

As shown in the figure, a UART frame typically consists of:

- one start bit,

- 5 to 8 data bits,

- an optional parity bit, and

- one or two stop bits.

In this project, the frame format is selected as 1 start bit, 8 data bits, no parity, and 1 stop bit (8N1), which is one of the most commonly used configurations.

The timing behavior of a UART transmission is presented in Figure 3.

Figure 3: UART timing diagram illustrating start bit detection and sampling of data bits at the center of each bit period.

During the idle state, the UART line is held at logic ‘1’. Communication begins when the transmitter pulls the line to logic ‘0’, indicating the start bit. After that, the data bits are transmitted starting from the least significant bit (LSB). The receiver detects the falling edge of the start bit and then samples the incoming data at the center of each bit period. After all data bits are received, the stop bit returns the line to logic ‘1’, marking the end of the frame.

To handle asynchronous timing, the receiver generally uses oversampling. In this project, the receiver samples the incoming data at 8 times the baud rate and determines each bit value by comparing the number of sampled logic ‘1’s and logic ‘0’s within one bit duration.

Figure 4 illustrates how parallel data is converted into serial form in the UART transmitter.

Figure 4: Parallel-to-serial data conversion performed by the UART transmitter.

The transmitter takes an 8-bit parallel input from the data bus and shifts it out bit by bit according to the baud rate. Similarly, the UART receiver performs the inverse operation by reconstructing the serial data stream back into an 8-bit parallel word.

The reverse operation is performed in the UART receiver, as illustrated in Figure 5. The receiver collects the incoming serial data bits from the RX line and reconstructs them into an 8-bit parallel word on the data bus. Each bit is sampled at the appropriate time instant, and after all data bits are received, the complete parallel data is made available at the output of the receiver.

Figüre 5: Serial-to-parallel data reconstruction performed by the UART receiver.

Overall, UART offers a simple and efficient method for serial communication with minimal wiring requirements. Its asynchronous nature and configurable frame structure make it suitable for a wide range of digital and embedded system applications.

- Algorithm

In this project, the UART driver is designed in a modular structure consisting of four main blocks: uart_tx, uart_rx, baud_gen, and uart_top. Each module is responsible for a specific task in the overall communication process.

The baud_gen module generates the required baud rate and its oversampled version from the system clock. This module provides timing references for both the transmitter and the receiver.

The uart_tx module implements the transmitter side of the UART protocol. It takes 8-bit parallel data as input and sends it serially according to the UART frame format using a finite state machine structure.

The uart_rx module performs the reverse operation. It receives serial data from the RX line, samples it using the oversampled baud clock, and reconstructs the original 8-bit parallel data at its output.

Finally, the uart_top module acts as the top-level integration block where the transmitter, receiver, and baud rate generator modules are instantiated and interconnected. It provides a single interface for external control signals and data input/output.

This modular design improves readability, reusability, and ease of debugging, while also allowing each block to be verified independently through simulation.

- Baud Rate Generator Module

The UART design requires two timing references: the baud rate clock used for bit-to-bit progression, and an oversampled clock used by the receiver for reliable sampling. Therefore, the baud rate generator is designed to produce baud and 8x baud timing pulses from the internal 100 MHz system clock.

The module is parameterized by the baud rate value (BAUD) and generates a one-clock-cycle pulse called tick_8x every:

where fclk=100 MHzand fbaudis selected as 115200 or 9600 in this project. Each time tick_8x is asserted, an internal modulo-8 counter is increased. When eight tick_8x pulses occur, the module produces a second one-clock-cycle pulse called tick_baud, which represents the actual baud timing.

An enable signal (en) is used to start/stop the divider operation. When en = 0, counters are cleared so that the timing restarts deterministically when enabled again. This behavior is especially useful for the receiver side, where sampling is aligned after detecting the start bit.

Verification:
The module is simulated for both baud rates. For 115200, the expected divider for the oversampled tick is:

For 9600, the expected value is:

In the simulation waveform, tick_8x appears periodically according to the computed divider value, and tick_baud is observed once for every eight tick_8x pulses, confirming correct operation.

  - Design Source

`timescale 1ns / 1ps

module baud_gen #(

parameter [16:0] freq = 115200 // Target Baud Rate

)

(

input  wire clk,               // System Clock

input  wire rst,               // Reset Signal (Active High)

input  wire baud_en,           // Module Enable Signal

output wire count_8x_ready,    // Pulse for 8x oversampling (Receiver)

output wire count_baud_ready   // Pulse for standard baud rate (Transmitter)

);

// Internal Constants

// System clock frequency set to 100 MHz

localparam integer clk_freq = 100_000_000;

// Internal Registers

reg count_8x_ready_reg;

reg count_baud_ready_reg;

// Counter for the 8x tick generation

// 14 bits are enough for standard baud rates at 100MHz

reg [13:0] counter_8x;

// Counter to divide 8x ticks down to 1x (counts 0 to 7)

reg [2:0] counter_baud;

always @(posedge clk) begin

// 1. Reset Logic

if (rst == 1'b1) begin

counter_baud         <= 3'b0;   // Reset baud counter

count_baud_ready_reg <= 1'b0;   // Clear baud output

counter_8x           <= 14'b0;  // Reset 8x counter

count_8x_ready_reg   <= 1'b0;   // Clear 8x output

end

// 2. Active Logic (When enabled)

else if (baud_en == 1'b1) begin

// Calculate limit: (Clock / (Baud * 8)) - 1

// Check if 8x counter reached the limit

if (counter_8x == ((clk_freq / (freq * 8)) - 1)) begin

counter_8x         <= 14'b0; // Reset counter

count_8x_ready_reg <= 1'b1;  // Generate 8x pulse

// Manage the Standard Baud Counter (0 to 7)

if (counter_baud == 3'd7) begin

counter_baud         <= 3'b0;

count_baud_ready_reg <= 1'b1; // Generate Baud pulse

end

else begin

counter_baud         <= counter_baud + 1'b1;

count_baud_ready_reg <= 1'b0;

end

end

else begin

// Increment 8x counter, keep outputs low

counter_8x           <= counter_8x + 1'b1;

count_8x_ready_reg   <= 1'b0;

count_baud_ready_reg <= 1'b0;

end

end

// 3. Disable Logic

else begin

count_8x_ready_reg   <= 1'b0;

counter_8x           <= 14'b0;

// FIXED: Originally 13'b0, changed to 3'b0 to match register width

counter_baud         <= 3'b0;

count_baud_ready_reg <= 1'b0;

end

end

// Assign internal registers to output ports

assign count_8x_ready   = count_8x_ready_reg;

assign count_baud_ready = count_baud_ready_reg;

endmodule

  - Testbench Code

`timescale 1ns / 1ps  // Simulation timescale: 1ns unit step with 1ps precision

module baud_gen_tb();

// Parameters

// 115200 requires 17 bits (2^17 = 131072), so [16:0] is sufficient.

parameter [16:0] freq = 115200;

// Testbench Signals

// Inputs to the DUT are defined as 'reg'

reg clk = 1'b0;

reg rst = 1'b0;

reg baud_en = 1'b0;      // Enable control signal

// Outputs from the DUT are defined as 'wire'

wire count_8x_ready;     // 8x oversampling clock pulse

wire count_baud_ready;   // Standard baud rate clock pulse

// Instantiate the DUT (Device Under Test)

baud_gen #(

.freq(freq)          // Pass the frequency parameter to the module

) uut (

.clk(clk),

.rst(rst),

.baud_en(baud_en),

.count_8x_ready(count_8x_ready),

.count_baud_ready(count_baud_ready)

);

// Clock Generation

// Generate a 100 MHz clock (Period = 10 ns).

// Toggles every 5 ns.

always begin

#5 clk = ~clk;

end

// Stimulus Process

initial begin

// 1. Initialize Inputs

rst = 1;             // Assert reset (active high)

baud_en = 0;         // Keep module disabled initially

// 2. Apply Reset

#10;                 // Wait for 10 ns

rst = 0;             // De-assert (release) reset

// 3. Enable Operation

baud_en = 1;         // Enable the baud rate generator

// 4. Run Simulation

#11000;              // Let it run for 11,000 ns (11 us)

// 5. Disable Operation

baud_en = 0;         // Disable the generator to verify stop behavior

// 6. Finish Simulation

#200;                // Wait a bit more for stability

$finish;             // Stop the simulation

end

endmodule

  - Simulation Waveform

For 115200Hz:

Figure 6: Simulation waveform of baud_gen for 115200 baud showing tick_8x and tick_baud pulses.

For 9600Hz:

Figure 7: Simulation waveform of baud_gen for 9600 baud showing tick_8x and tick_baud pulses.

- UART Transmitter

The uart_tx module is responsible for transmitting an 8-bit parallel input as a serial UART frame. The frame format used in this project is 1 start bit, 8 data bits (LSB first), and 1 stop bit. The transmitter is implemented using a finite state machine (FSM) with four states: IDLE, START, DATA, STOP.

- IDLE: The TX line is held at logic ‘1’. The module waits for tx_en to become high. When tx_en is asserted, the input byte is latched into an internal register and the baud generator is enabled (baud_en = 1). A one-cycle start pulse is generated to indicate the beginning of a transmission.

- START: The transmitter drives the line low (tx_out = 0) for exactly one baud period to send the start bit.

- DATA: The transmitter sends the data bits sequentially. The least significant bit is transmitted first. The bit progression is controlled by the tick_baud signal (a one-cycle pulse from the baud rate generator). At each tick_baud, the next bit is output and the internal shift register is updated. After 8 bits, the FSM transitions to STOP.

- STOP: The line is driven high (tx_out = 1) for one baud period as the stop bit. After that, the transmitter disables the baud generator, generates a one-cycle done pulse, and returns to IDLE.

During the transmission process, the busy signal remains high to indicate that the module is actively sending a frame.

  - Design Source

`timescale 1ns / 1ps

module uart_tx #(

parameter [16:0] freq = 115200 // Baud rate parameter

)

(

input wire clk,             // System clock

input wire rst,             // Reset (Active High)

input wire [7:0] d_in,      // Data to be transmitted

input wire tx_en,           // Start transmission signal

output wire seri_out,       // Serial data output (Tx pin)

output reg start,           // Status: Start bit being sent

output reg busy,            // Status: Module is busy

output wire done            // Status: Transmission complete

);

// Internal Signals

wire count_baud_ready;      // Baud tick from generator

wire out_of_use;            // Unused 8x tick

reg done_reg;               // Register for done signal

reg seri_out_reg;           // Register for serial output

reg baud_en;                // Enable signal for baud generator

reg [2:0] bit_in;           // Bit counter (0 to 7)

reg [7:0] buffer;           // Data buffer (Shift register)

// State Machine Definitions

localparam IDLE  = 2'b00;

localparam START = 2'b01;

localparam DATA  = 2'b10;

localparam STOP  = 2'b11;

reg [1:0] state;

// Instantiate Baud Rate Generator

baud_gen #(

.freq(freq)

) divider (

.clk(clk),

.rst(rst),

.baud_en(baud_en),

.count_8x_ready(out_of_use), // Not needed for TX

.count_baud_ready(count_baud_ready)

);

// Main State Machine

always @(posedge clk) begin

if (rst == 1'b1) begin

state        <= IDLE;

done_reg     <= 0;

start        <= 0;

busy         <= 0;

seri_out_reg <= 1; // Idle state is High (1)

baud_en      <= 0;

bit_in       <= 0;

buffer       <= 8'b0;

end

else begin

case(state)

// State 0: Wait for tx_en signal

IDLE: begin

done_reg     <= 0;

seri_out_reg <= 1; // Tx line High when idle

baud_en      <= 0;

busy         <= 0;

start        <= 0;

if (tx_en == 1) begin

state  <= START;

buffer <= d_in; // Load data into buffer

bit_in <= 0;    // Reset bit counter

end

end

// State 1: Send Start Bit (Low)

START: begin

start        <= 1;

seri_out_reg <= 0; // Start bit is always 0

baud_en      <= 1; // Turn on baud generator

busy         <= 1;

// Wait for one baud period

if(count_baud_ready == 1) begin

state <= DATA;

end

end

// State 2: Send Data Bits (LSB First)

DATA: begin

start        <= 0;

busy         <= 1;

seri_out_reg <= buffer[0]; // Send LSB

if(count_baud_ready == 1) begin

if(bit_in == 7) begin

state  <= STOP;

bit_in <= 0;

end

else begin

buffer <= buffer >> 1; // Shift right to get next bit

bit_in <= bit_in + 1;

end

end

end

// State 3: Send Stop Bit (High)

STOP: begin

seri_out_reg <= 1; // Stop bit is always 1

if(count_baud_ready == 1) begin

state    <= IDLE;

baud_en  <= 0; // Turn off baud generator

busy     <= 0;

done_reg <= 1; // Signal completion

end

end

default: begin

state <= IDLE;

end

endcase

end

end

// Output assignments

assign done = done_reg;

assign seri_out = seri_out_reg;

endmodule

  - Finite State Machine

  - Simulation Source

module uart_tx_tb;

// Parameters

parameter [16:0] freq = 115200;

// Testbench Signals

reg clk = 1'b0;

reg rst = 1'b0;

reg [7:0] d_in;

reg tx_en = 1'b0;

// Outputs from DUT

wire seri_out;

wire done;

wire start; // Added to match DUT

wire busy;  // Added to match DUT

// File I/O Variables

integer file, r;

reg [7:0] stimulus_data;

integer i;

// Instantiate the Unit Under Test (UUT)

uart_tx #(

.freq(freq)

) uut (

.clk(clk),

.rst(rst),

.d_in(d_in),

.tx_en(tx_en),

.seri_out(seri_out),

.start(start), // Connected for observation

.busy(busy),   // Connected for observation

.done(done)

);

// Clock Generation

// 100 MHz clock (10ns period -> toggle every 5ns)

always #5 clk = ~clk;

// Main Stimulus Process

initial begin

// 1. Open the file

// Ensure "stimulus2.txt" exists in the simulation directory

file = $fopen("stimulus2.txt", "r");

if (file == 0) begin

$display("Error: Cannot open stimulus2.txt");

$finish;

end

// 2. Initialize and Reset

rst = 1;

d_in = 8'b0;

tx_en = 0;

#8000; // Wait 8us (long reset)

rst = 0; // Release reset

// 3. Read loop

// Reads 4 lines of binary data from the file

for (i = 0; i < 4; i = i + 1) begin

// Read binary data (%b) into variable

r = $fscanf(file, "%b\n", stimulus_data);

if (r == 1) begin // If read was successful

// Drive new data

d_in = stimulus_data;

// Generate Enable Pulse (1 clock cycle)

tx_en = 1;

#10;

tx_en = 0;

// Wait for transmission to complete

wait (done == 1);

// Wait before sending the next byte

#15000;

end

end

// 4. Cleanup

$fclose(file);

$finish;

end

endmodule

I  write 4 data into a text file as each one 8 bits. I used them in the simulation and checked it.

Here they are:
00101011

11000100

11110011

01001110

  - Simulation Waveform

Figure 8: UART transmitter simulation waveform for input bytes 0x2B, 0xC4, 0xF3 and 0x4E, showing start bit, data bits (LSB first) and stop bit together with control signals.

2b = 0010 1011₂ → LSB first: 1 1 0 1 0 1 0 0

c4 = 1100 0100₂ → LSB first: 0 0 1 0 0 0 1 1

f3 = 1111 0011₂ → LSB first: 1 1 0 0 1 1 1 1

4e = 0100 1110₂ → LSB first: 0 1 1 1 0 0 1 0

In the transmitter simulation, the input bytes 0x2B, 0xC4, 0xF3 and 0x4E are applied through the stimulus file. As shown in Figure X, each value is transmitted with a start bit, followed by 8 data bits in LSB-first order, and a stop bit. The busy signal stays high during the frame, while done pulses at the end of each transmission, confirming the correct operation of the UART transmitter.

- UART Receiver

  - Design Source

module uart_rx #(

parameter [16:0] freq = 115200

)

(

input wire clk,             // System Clock

input wire rst,             // Reset (Active High)

input wire d_in,            // Serial Data Input (Rx Pin)

input wire rx_en,           // Receiver Enable Signal

output wire [7:0] d_out,    // Parallel Data Output

output reg start,           // Status: Start bit detected

output reg busy,            // Status: Receiver is busy

output wire done            // Status: Reception complete

);

// Internal Registers

reg [3:0] count_ones;       // Counter for logic '1' samples

reg [3:0] count_zeros;      // Counter for logic '0' samples

reg [2:0] counter_halfperiod; // Counts to find center of start bit

reg [7:0] d_out_reg;        // Shift register for data

reg [3:0] bit_in;           // Bit index counter (0-7)

reg done_reg;               // Internal done signal

reg baud_en;                // Control for Baud Generator

// Wires

wire count_baud_ready;      // Full bit period tick

wire count_8x_ready;        // 8x oversampling tick

// State Machine Definitions

localparam IDLE  = 2'b00;

localparam START = 2'b01;

localparam DATA  = 2'b10;

localparam STOP  = 2'b11;

reg [1:0] state;

// Instantiate Baud Rate Generator

baud_gen #(

.freq(freq)

) divider (

.clk(clk),

.rst(rst),

.baud_en(baud_en),

.count_8x_ready(count_8x_ready),

.count_baud_ready(count_baud_ready)

);

// Main State Machine

always @(posedge clk) begin

if (rst == 1'b1) begin

state              <= IDLE;

done_reg           <= 0;

start              <= 0;

busy               <= 0;

baud_en            <= 0;

counter_halfperiod <= 3'b0;

count_ones         <= 0;

count_zeros        <= 0;

d_out_reg          <= 8'b0;

bit_in             <= 0;

end

else begin

case(state)

// State 0: Wait for Enable

IDLE: begin

done_reg <= 0;

baud_en  <= 0;

busy     <= 0;

start    <= 0;

if (rx_en == 1) begin

state  <= START;

bit_in <= 0;

start  <= 1; // Indicate ready to receive

end

end

// State 1: Detect Start Bit (Logic 0)

START: begin

// Verify that line is Low (Start Bit)

if(d_in == 0) begin

baud_en <= 1; // Enable counters

// Wait for half a bit period (approx 4 ticks of 8x clock)

// to align with the center of the pulse.

if(counter_halfperiod < 4) begin

if(count_8x_ready == 1) begin

counter_halfperiod <= counter_halfperiod + 1;

end

end

else begin

// Middle of start bit reached

state              <= DATA;

busy               <= 1;

start              <= 0;

counter_halfperiod <= 3'b0;

// Important: Disable baud_gen momentarily.

// This resets the baud_gen counters so that when we

// re-enable it in DATA state, it starts counting fresh

// for exact bit alignment.

baud_en            <= 0;

end

end

else begin

// If line goes High unexpectedly, it might be noise

// but current logic stays in START if rx_en was high.

baud_en <= 0;

counter_halfperiod <= 0;

end

end

// State 2: Receive 8 Data Bits

DATA: begin

baud_en <= 1;

// End of a bit period?

if(count_baud_ready == 1) begin

// Majority Voting Logic

if(count_ones > count_zeros) begin

// Shift in a '1' (LSB first reception)

d_out_reg <= {1'b1, d_out_reg[7:1]};

end

else begin

// Shift in a '0'

d_out_reg <= {1'b0, d_out_reg[7:1]};

end

// Reset counters for next bit

count_ones  <= 3'b0;

count_zeros <= 3'b0;

// Check if all 8 bits are received

if(bit_in == 7) begin

busy    <= 0;

state   <= STOP;

bit_in  <= 0;

end

else begin

bit_in <= bit_in + 1;

end

end

// Sampling Phase (8x oversampling)

else begin

if(count_8x_ready == 1) begin

if(d_in == 1) begin

count_ones <= count_ones + 1;

end

else begin

count_zeros <= count_zeros + 1;

end

end

end

end

// State 3: Stop Bit

STOP: begin

// Wait for stop bit duration

if(count_baud_ready == 1) begin

state    <= IDLE;

baud_en  <= 0;

done_reg <= 1; // Signal Valid Data Available

end

end

default: begin

state     <= IDLE;

done_reg  <= 0;

baud_en   <= 0;

end

endcase

end

end

// Output Assignments

assign done  = done_reg;

assign d_out = d_out_reg;

endmodule

The UART receiver is designed as a finite state machine with IDLE, START, DATA and STOP states. Since the receiver was implemented before the top module, the baud rate generator is instantiated inside the receiver to provide the required baud and 8x baud timing signals. In the final top-level design, this module is later moved and shared at the top level.

In the IDLE state, the receiver waits until rx_en becomes logic high. When enabled and a falling edge is detected on the input, the FSM switches to the START state. In this state, the receiver waits for half of the baud period (T/2) to align sampling to the center of the start bit. This half-bit delay is measured by counting four ticks of the 8x baud clock.

After T/2, the FSM enters the DATA state. Here, the input d_in is sampled eight times for each bit using the 8x baud frequency. The number of logic ones and zeros is compared, and the bit value is decided by majority voting. After all eight data bits are received, as indicated by the bit counter, the FSM switches to the STOP state.

In the STOP state, the receiver waits for one baud period to validate the stop bit (logic ‘1’). Finally, the FSM returns to IDLE and a one-cycle done pulse is generated to indicate that a valid byte is available at the output.

  - Finite State Machine

  - Simulation Source

module uart_rx_tb;

// Parameters

parameter freq = 115200;

// Bit period for 115200 baud: 1,000,000,000 / 115200 = 8680 ns

localparam BIT_PERIOD = 8680;

// Testbench Signals

reg clk = 0;

reg rst = 0;

reg d_in = 1; // Rx line is IDLE High

reg rx_en = 0;

// Outputs from DUT

wire [7:0] d_out;

wire done;

wire start; // Added to match DUT

wire busy;  // Added to match DUT

// Test Variables

reg [7:0] test_data;

reg [7:0] expected_data;

integer file, i, r;

// Clock Generation (100 MHz, 10ns period)

always #5 clk = ~clk;

// Instantiate the Unit Under Test (UUT)

uart_rx #(

.freq(freq)

) uut (

.clk(clk),

.rst(rst),

.d_in(d_in),

.rx_en(rx_en),

.d_out(d_out),

.start(start), // Connected for observation

.busy(busy),   // Connected for observation

.done(done)

);

// Task: Simulate UART Serial Transmission

// Sends one byte of data serially on 'd_in' line

task send_uart_byte(input [7:0] para_in);

integer k;

begin

// 1. Send Start Bit (Low)

d_in = 0;

#BIT_PERIOD;

// 2. Send Data Bits (LSB to MSB)

for (k = 0; k < 8; k = k + 1) begin

d_in = para_in[k];

#BIT_PERIOD;

end

// 3. Send Stop Bit (High)

d_in = 1;

#BIT_PERIOD;

end

endtask

// Main Test Sequence

initial begin

// 1. Open File

file = $fopen("stimulus2.txt", "r");

if (file == 0) begin

$display("Error: Cannot open stimulus2.txt");

$finish;

end

$display("Starting UART RX testbench...");

// 2. Reset System

rst = 1;

#10000;

rst = 0;

rx_en = 0;

#45;

i = 0;

// 3. Read loop

// Uses $fscanf return value 'r' to ensure safe file reading

while (!$feof(file)) begin

r = $fscanf(file, "%b\n", test_data);

if (r == 1) begin // Process only if read was successful

expected_data = test_data;

// Enable Receiver

rx_en = 1;

// Send Data via Task

send_uart_byte(test_data);

// Wait a bit to ensure 'done' signal is caught

#100;

// Check Result

if (d_out == expected_data) begin

$display("Test %0d Passed! Sent: %b, Received: %b", i + 1, expected_data, d_out);

end else begin

$display("Test %0d Failed! Sent: %b, Received: %b", i + 1, expected_data, d_out);

end

// Disable Receiver briefly between bytes

rx_en = 0;

#60;

i = i + 1;

end

end

$display("Test completed!");

$fclose(file);

$finish;

end

endmodule

The UART receiver is verified using a self-checking testbench. The purpose of the testbench is to apply serial UART frames to the receiver input and to automatically check whether the reconstructed parallel output matches the expected values.

First, a 100 MHz clock is generated to drive the design under test. After reset is released, the receiver is enabled by asserting the rx_en signal. The baud rate parameter is set according to the selected baud value used in the design.

The testbench reads a sequence of input bytes from an external text file using file I/O system tasks. Each byte is stored in a variable and is then transmitted serially to the receiver input (d_in) by means of a dedicated task. This task generates a valid UART frame by:

- driving the line low for one bit period to represent the start bit,

- sending the 8 data bits in LSB-first order, each for one bit period, and

- driving the line high for one bit period to generate the stop bit.

During reception, the testbench waits for the done signal from the receiver, which indicates that a full byte has been reconstructed. The received output d_out is then compared against the expected value read from the file. If both values match, a “Passed” message is printed to the simulation console; otherwise, a “Failed” message is reported together with the sent and received data.

This approach allows the receiver to be tested automatically for multiple input patterns without manual inspection of the waveform. The simulation results confirm that the UART receiver correctly detects the start bit, performs oversampling, reconstructs the parallel data, and asserts the done signal after each valid frame.

  - Simulation Waveform

Figure 9: UART receiver simulation waveform showing start-bit detection, T/2 alignment, 8x oversampling during data reception, and the reconstructed parallel output d_out with done pulses.

In the receiver simulation, serial data frames are applied to the d_in input using the testbench, while rx_en is asserted. As shown in Figure X, the receiver detects the falling edge of the start bit and waits for half a bit duration (T/2) to align sampling to the bit center. Then, in the DATA state, the input is sampled eight times per bit and the bit value is determined by majority voting.

After eight data bits are reconstructed, the receiver checks the stop bit and produces a one-cycle done pulse. The parallel output d_out matches the expected values (0x2B, 0xC4, 0xF3, and 0x4E), verifying correct operation of the UART receiver.

- TOP Module

  - Design Source

module uart_top #(

parameter [16:0] BAUD = 115200

)(

input  wire       clk,          // 100 MHz system clock

input  wire       rst,          // active-high reset

input  wire [7:0] tx_data,      // byte to be transmitted

input  wire       tx_en,         // start TX

input  wire       rx_en,         // enable RX

output wire [7:0] rx_data,      // received byte

output wire       tx_done,

output wire       rx_done,

output wire       tx_start,

output wire       rx_start,

output wire       tx_busy,

output wire       rx_busy,

output wire       tx_out         // exposed serial line (loopback)

);

// Loopback serial line: TX output is directly connected to RX input

wire uart_line;

// -------------------------

// Transmitter Instance (TX)

// -------------------------

// baud_gen is instantiated inside uart_tx in this version

uart_tx #(

.freq(BAUD)

) u_tx (

.clk     (clk),

.rst     (rst),

.d_in    (tx_data),

.tx_en   (tx_en),

.seri_out(uart_line),

.start   (tx_start),

.busy    (tx_busy),

.done    (tx_done)

);

// -------------------------

// Receiver Instance (RX)

// -------------------------

// baud_gen is instantiated inside uart_rx in this version

uart_rx #(

.freq(BAUD)

) u_rx (

.clk  (clk),

.rst  (rst),

.d_in (uart_line),

.rx_en(rx_en),

.d_out(rx_data),

.start(rx_start),

.busy (rx_busy),

.done (rx_done)

);

// Expose the internal serial line for observation in simulation

assign tx_out = uart_line;

endmodule

The uart_top module integrates the UART transmitter and receiver into a single loopback structure. The transmitter output is directly wired to the receiver input through an internal serial line, enabling end-to-end verification of UART transmission and reception. Separate enable signals are used to control transmission (tx_en) and reception (rx_en).

In this implementation, the baud rate generation is handled internally by the uart_tx and uart_rx modules (each module includes its own baud generator logic). The receiver uses an oversampled timing reference (8x baud) for reliable sampling, while the transmitter uses the baud timing to advance the frame bits. This top-level integration allows verifying the UART link by transmitting a byte and checking that the same byte is reconstructed at the receiver output.

  - Simulation Source

module uart_top_tb;

// -------------------------

// Parameters

// -------------------------

parameter [16:0] BAUD = 115200;

// -------------------------

// TB Signals

// -------------------------

reg        clk = 1'b0;

reg        rst = 1'b0;

reg [7:0]  tx_data;

reg        tx_en = 1'b0;

reg        rx_en = 1'b0;

wire [7:0] rx_data;

wire       tx_done;

wire       rx_done;

wire       tx_busy;

wire       rx_busy;

wire       tx_start;

wire       rx_start;

wire       tx_out;

// -------------------------

// File I/O & stimulus storage

// -------------------------

integer k;

integer fd;

integer rc;

reg [7:0] stim_mem [0:3];

// -------------------------

// DUT: UART loopback top

// -------------------------

uart_top #(

.BAUD(BAUD)

) dut (

.clk       (clk),

.rst       (rst),

.tx_data   (tx_data),

.tx_en     (tx_en),

.rx_en     (rx_en),

.rx_data   (rx_data),

.tx_done   (tx_done),

.rx_done   (rx_done),

.tx_start  (tx_start),

.rx_start  (rx_start),

.tx_busy   (tx_busy),

.rx_busy   (rx_busy),

.tx_out    (tx_out)

);

// -------------------------

// 100 MHz clock

// -------------------------

always #5 clk = ~clk;

// -------------------------

// Main test sequence

// -------------------------

initial begin

// 1) Read stimulus file (4 bytes)

fd = $fopen("stimulus2.txt", "r");

if (fd == 0) begin

$display("ERROR: stimulus2.txt could not be opened.");

$finish;

end

for (k = 0; k < 4; k = k + 1) begin

rc = $fscanf(fd, "%b\n", stim_mem[k]);

end

$fclose(fd);

// 2) Reset sequence

$display("Starting UART Loopback Test...");

rst = 1'b1;

#20000;

rst   = 1'b0;

tx_en = 1'b0;

rx_en = 1'b0;

// 3) Run tests

for (k = 0; k < 4; k = k + 1) begin

// Apply new input byte

tx_data = stim_mem[k];

// Enable RX before triggering TX

rx_en = 1'b1;

// Start TX (1 clk pulse is enough)

tx_en = 1'b1;

#10;

tx_en = 1'b0;

// Wait for RX completion

wait (rx_done == 1'b1);

// Give one extra clk edge to avoid sampling an old rx_data value

@(posedge clk);

// Check results

if (rx_data == stim_mem[k]) begin

$display("Test %0d PASS: Sent=%b (%h), Received=%b (%h)",

k+1, stim_mem[k], stim_mem[k], rx_data, rx_data);

end else begin

$display("Test %0d FAIL: Sent=%b (%h), Received=%b (%h)",

k+1, stim_mem[k], stim_mem[k], rx_data, rx_data);

end

// Prepare for next test

rx_en = 1'b0;

#20000;

end

$display("All tests completed.");

$finish;

end

endmodule

The uart_top_tb testbench generates the system clock and reset, reads stimulus bytes from a text file, and applies them to the UART transmitter. By enabling the receiver and triggering the transmitter, the testbench verifies the loopback connection between TX and RX. For each transmitted frame, the receiver output is compared against the expected value and pass/fail messages are reported, enabling automated functional verification of the complete UART system.

  - Simulation Waveform

    - Behavioral Simulation:

Figure 13: Behavioral simulation waveform of the UART top module showing loopback transmission and reception of four bytes with corresponding tx_done and rx_done pulses.

Figure 13 shows the behavioral simulation results of the uart_top module configured in loopback mode. The input bytes read from the stimulus file are applied to the transmitter through tx_data while tx_en is asserted. For each test case, the transmitter generates a UART frame consisting of a start bit, eight data bits, and a stop bit, which appears on the serial line tx_out. This serial stream is directly fed into the receiver input. When a complete frame is received, the receiver reconstructs the parallel byte at rx_data and asserts the rx_done signal. The waveform demonstrates that for all transmitted values (0x2B, 0xC4, 0xF3, and 0x4E), the received data matches the transmitted data, confirming correct end-to-end operation of the UART loopback system.

    - Post-implementation Timing Simulation

Figure 14: Post-implementation timing simulation of the UART top module showing correct loopback operation with gate-level delays.

Figure 14 shows the post-implementation timing simulation results of the uart_top module after place-and-route. In this simulation, the synthesized and routed netlist is used together with timing delays, providing a realistic representation of hardware behavior. Despite the presence of propagation delays, the transmitter successfully generates UART frames on the serial line tx_out, and the receiver correctly reconstructs the transmitted bytes at rx_data. The tx_done and rx_done signals are asserted for each frame, indicating proper completion of transmission and reception. The results demonstrate that the UART top module operates correctly not only at the behavioral level but also after implementation with actual timing effects.