ISTANBUL TECHNICAL UNIVERSITY

FACULTY OF ELECTRICAL – ELECTRONICS

Digital System Design Applications

Project-2

Class

Class Lecturer: Sıddıka Berna Örs Yalçın

Class Assistants: Bilgi Görkem Yazgaç, Deniz Zakir Eroğlu

Term: 2025/2026 Fall Term

Student

Name-Surname: Mehmet Emin Delibaş

No: 040200092

E-mail: delibas20@itu.edu.tr

Due Date: 27/12/2025

UART PROTOCOL

- Explanation

Universal Asynchronous Receiver Transmitter (UART) is a widely used hardware communication protocol that enables asynchronous serial data transfer between digital devices. Unlike synchronous communication methods, UART does not use a shared clock signal between the transmitter and the receiver. Instead, both sides operate with a predefined baud rate, which determines the duration of each transmitted bit.

Figure 1: Physical UART connection between two devices showing TX, RX and common ground lines.[1]

Figure 1 shows the basic physical connection of two UART-enabled devices. Each device has a transmit (TX) and a receive (RX) line. The TX pin of one device is connected to the RX pin of the other device, while both systems share a common ground (GND). This cross-connection allows full-duplex communication between the two devices.

UART communication is based on framing the data with control bits. A general UART frame structure is illustrated in Figure 2.

Figure 2: General UART frame structure including start bit, data bits, optional parity bit and stop bit(s).[2]

As shown in the figure, a UART frame typically consists of:

- one start bit,

- 5 to 8 data bits,

- an optional parity bit, and

- one or two stop bits.

In this project, the frame format is selected as 1 start bit, 8 data bits, no parity, and 1 stop bit (8N1), which is one of the most commonly used configurations.

The timing behavior of a UART transmission is presented in Figure 3.

Figure 3: UART timing diagram illustrating start bit detection and sampling of data bits at the center of each bit period.[3]

During the idle state, the UART line is held at logic ‘1’. Communication begins when the transmitter pulls the line to logic ‘0’, indicating the start bit. After that, the data bits are transmitted starting from the least significant bit (LSB). The receiver detects the falling edge of the start bit and then samples the incoming data at the center of each bit period. After all data bits are received, the stop bit returns the line to logic ‘1’, marking the end of the frame.

To handle asynchronous timing, the receiver generally uses oversampling. In this project, the receiver uses an 8x baud tick as a timing reference and samples the serial input once per bit period after alignment to reconstruct each data bit.

Figure 4 illustrates how parallel data is converted into serial form in the UART transmitter.

Figure 4: Parallel-to-serial data conversion performed by the UART transmitter.[4]

The transmitter takes an 8-bit parallel input from the data bus and shifts it out bit by bit according to the baud rate. Similarly, the UART receiver performs the inverse operation by reconstructing the serial data stream back into an 8-bit parallel word.

The reverse operation is performed in the UART receiver, as illustrated in Figure 5. The receiver collects the incoming serial data bits from the RX line and reconstructs them into an 8-bit parallel word on the data bus. Each bit is sampled at the appropriate time instant, and after all data bits are received, the complete parallel data is made available at the output of the receiver.

Figure 5: Serial-to-parallel data reconstruction performed by the UART receiver.[5]

Overall, UART offers a simple and efficient method for serial communication with minimal wiring requirements. Its asynchronous nature and configurable frame structure make it suitable for a wide range of digital and embedded system applications.

- Algorithm

In this project, the UART driver is designed in a modular structure consisting of four main blocks: uart_tx, uart_rx, baud_gen, and uart_top. Each module is responsible for a specific task in the overall communication process.

The baud_gen module generates the required baud rate and its oversampled version from the system clock. This module provides timing references for both the transmitter and the receiver.

The uart_tx module implements the transmitter side of the UART protocol. It takes 8-bit parallel data as input and sends it serially according to the UART frame format using a finite state machine structure.

The uart_rx module performs the reverse operation. It receives serial data from the RX line, samples it using the oversampled baud clock, and reconstructs the original 8-bit parallel data at its output.

Finally, the uart_top module acts as the top-level integration block where the transmitter, receiver, and baud rate generator modules are instantiated and interconnected. It provides a single interface for external control signals and data input/output.

This modular design improves readability, reusability, and ease of debugging, while also allowing each block to be verified independently through simulation.

- Baud Rate Generator Module

The UART design requires two timing references: the baud rate clock used for bit-to-bit progression, and an oversampled clock used by the receiver for reliable sampling. Therefore, the baud rate generator is designed to produce baud and 8x baud timing pulses from the internal 100 MHz system clock.

The module is parameterized by the baud rate value (BAUD) and generates a one-clock-cycle pulse called tick_8x every:

where fclk=100 MHzand fbaudis selected as 115200 or 9600 in this project. Each time tick_8x is asserted, an internal modulo-8 counter is increased. When eight tick_8x pulses occur, the module produces a second one-clock-cycle pulse called tick_baud, which represents the actual baud timing.

An enable signal (en) is used to start/stop the divider operation. When en = 0, counters are cleared so that the timing restarts deterministically when enabled again. This behavior is especially useful for the receiver side, where sampling is aligned after detecting the start bit.

Verification:
The module is simulated for both baud rates. For 115200, the expected divider for the oversampled tick is:

For 9600, the expected value is:

In the simulation waveform, tick_8x appears periodically according to the computed divider value, and tick_baud is observed once for every eight tick_8x pulses, confirming correct operation.

  - Design Source

// Baud rate generator module

// Generates tick pulses for 8x oversampling and baud rate timing

module baud_gen #(parameter [16:0] freq = 115200)

(

input  clk,               // System clock (100 MHz)

input  rst,               // Active-high synchronous reset

input  baud_en,           // Enable signal for baud generator

output count_8x_ready,    // Pulse indicating 8x baud tick

output count_baud_ready   // Pulse indicating baud tick (1x)

);

// Internal clock frequency in Hz

localparam integer clk_freq = 100000000; // 100 MHz

// Registers to hold output pulses

reg count_8x_ready_reg;

reg count_baud_ready_reg;

// Counter for generating 8x baud ticks

reg [13:0] counter_8x;

// Counter for dividing 8x ticks into 1 baud tick

reg [2:0] counter_baud;

// Sequential logic driven by system clock

always @(posedge clk) begin

if (rst == 1'b1) begin

// Reset all counters and outputs

counter_baud <= 3'b0;           // Baud tick counter reset

count_baud_ready_reg <= 1'b0;   // Clear baud tick pulse

counter_8x <= 13'b0;             // 8x tick counter reset

count_8x_ready_reg <= 1'b0;     // Clear 8x tick pulse

end

else if (baud_en == 1'b1) begin

// Baud generator active

// Check if 8x counter reached terminal count

if (counter_8x == ((clk_freq/(freq*8)) - 1)) begin

counter_8x <= 13'b0;         // Reset 8x counter

count_8x_ready_reg <= 1'b1; // Generate 8x tick pulse

// Count 8 pulses to generate one baud tick

if (counter_baud == 3'd7) begin

counter_baud <= 3'b0;           // Reset baud counter

count_baud_ready_reg <= 1'b1;  // Generate baud tick pulse

end

else begin

counter_baud <= counter_baud + 1'b1; // Increment baud counter

count_baud_ready_reg <= 1'b0;        // No baud pulse yet

end

end

else begin

// Continue counting towards next 8x tick

counter_8x <= counter_8x + 1'b1;

count_8x_ready_reg <= 1'b0;

count_baud_ready_reg <= 1'b0;

end

end

else begin

// When baud generator is disabled, clear counters and outputs

count_8x_ready_reg <= 1'b0;

counter_8x <= 13'b0;

counter_baud <= 3'b0;

count_baud_ready_reg <= 1'b0;

end

end

// Assign registered outputs to module outputs

assign count_8x_ready   = count_8x_ready_reg;

assign count_baud_ready = count_baud_ready_reg;

endmodule

The baud_gen module generates timing pulses for UART operation by dividing the 100 MHz system clock to the desired baud rate given by the parameter freq. The counter counter_8x produces an oversampling pulse (count_8x_ready) at eight times the baud rate. A second counter, counter_baud, counts these pulses and generates a baud-rate pulse (count_baud_ready) every eight oversampling ticks.

The module operates only when the enable signal baud_en is asserted. When baud_en is low or when reset is active, all counters and outputs are cleared. This structure provides accurate baud and oversampling timing for the UART transmitter and receiver.

  - Testbench Code

// Testbench for the baud rate generator module

// This testbench verifies the generation of 8x and baud-rate tick pulses

module baud_gen_tb();

// Baud rate parameter (17 bits are enough to represent 115200)

parameter [16:0] freq = 115200;

// Testbench signals

reg clk = 1'b0;          // System clock

reg rst = 1'b0;          // Reset signal

reg baud_en = 1'b0;      // Enable signal for baud generator

// Outputs from DUT

wire count_8x_ready;     // 8x oversampling tick pulse

wire count_baud_ready;   // Baud-rate tick pulse

// Instantiate the DUT (Device Under Test)

baud_gen #(

.freq(freq)

) uut (

.clk(clk),                       // Connect clock

.rst(rst),                       // Connect reset

.baud_en(baud_en),               // Connect enable

.count_8x_ready(count_8x_ready), // Connect 8x tick output

.count_baud_ready(count_baud_ready) // Connect baud tick output

);

// Generate a 100 MHz clock (10 ns period, toggle every 5 ns)

always begin

#5 clk = ~clk;

end

// Stimulus process

initial begin

// Initial state: apply reset and keep module disabled

rst = 1'b1;

baud_en = 1'b0;

// Wait for 10 ns and deassert reset

#10;

rst = 1'b0;

// Enable the baud generator

baud_en = 1'b1;

// Let the generator run long enough to observe output pulses

#11000;

// Disable the baud generator

baud_en = 1'b0;

// Wait for a short time and finish the simulation

#200;

$finish;

end

endmodule

The baud_gen_tb module is a testbench designed to verify the functionality of the baud_gen module. It generates a 100 MHz clock by toggling the clk signal every 5 ns and applies reset and enable stimuli to the design under test.

At the beginning of the simulation, the reset signal rst is asserted to initialize the internal counters of the baud generator. After a short delay, the reset is deasserted and the enable signal baud_en is set high to activate the baud generator. The testbench then allows the module to run for a sufficient duration so that both the oversampling tick (count_8x_ready) and the baud-rate tick (count_baud_ready) can be observed.

After this interval, the enable signal is deasserted to verify that the module stops generating output pulses when disabled. Finally, the simulation is terminated. This testbench confirms that the baud generator produces correct timing pulses and responds properly to reset and enable control signals.

  - Simulation Waveform

For 115200Hz:

Figure 6: Waveform of the baud_gen module showing generation of count_8x_ready and count_baud_ready pulses for a baud rate of 115200 bps under a 100 MHz clock..

For 9600Hz:

Figure 7: Simulation waveform of baud_gen for 9600 baud.

  - Post-implementation Timing Simulation

Figure 8: Post-implementation timing simulation waveform of the baud rate generator, showing the generation of count_8x_ready and count_baud_ready pulses synchronized to the 100 MHz system clock, confirming correct baud and oversampling tick timing under implementation delays.

  - Utilization & Timing Summary Report

- UART Transmitter

The uart_tx module is responsible for transmitting an 8-bit parallel input as a serial UART frame. The frame format used in this project is 1 start bit, 8 data bits (LSB first), and 1 stop bit. The transmitter is implemented using a finite state machine (FSM) with four states: IDLE, START, DATA, STOP.

- IDLE: The TX line is held at logic ‘1’. The module waits for tx_en to become high. When tx_en is asserted, the input byte is latched into an internal register and the baud generator is enabled (baud_en = 1). A one-cycle start pulse is generated to indicate the beginning of a transmission.

- START: The transmitter drives the line low (tx_out = 0) for exactly one baud period to send the start bit.

- DATA: The transmitter sends the data bits sequentially. The least significant bit is transmitted first. The bit progression is controlled by the tick_baud signal (a one-cycle pulse from the baud rate generator). At each tick_baud, the next bit is output and the internal shift register is updated. After 8 bits, the FSM transitions to STOP.

- STOP: The line is driven high (tx_out = 1) for one baud period as the stop bit. After that, the transmitter disables the baud generator, generates a one-cycle done pulse, and returns to IDLE.

During the transmission process, the busy signal remains high to indicate that the module is actively sending a frame.

  - Design Source

The uart_tx module implements a UART transmitter using a finite state machine. It serializes 8-bit parallel input data into a standard UART frame consisting of one start bit, eight data bits (LSB first), and one stop bit. The transmission is synchronized with an external baud rate generator through the tx_count_baud_ready signal. Control signals start, busy, and done indicate the transmission status.

module uart_tx #(

parameter [16:0] freq = 115200   // Baud rate parameter (not used directly here)

)(

input  wire       clk,           // System clock

input  wire       rst,           // Synchronous reset

input  wire [7:0] d_in,          // Parallel data to transmit

input  wire       tx_en,         // Transmit enable signal

// Interface to baud rate generator

input  wire       tx_count_baud_ready, // Baud tick: one pulse per bit time

output reg        tx_baud_en,           // Enable signal for baud generator

output wire       seri_out,      // UART serial output line

output reg        start,         // Indicates start of transmission

output reg        busy,          // High while transmitting

output wire       done           // One-cycle pulse when transmission completes

);

reg done_reg;                    // Internal done flag

reg seri_out_reg;               // Registered serial output

reg [2:0] bit_in;                // Bit counter (0 to 7)

reg [7:0] buffer;                // Shift register for transmit data

// FSM state encoding

localparam IDLE  = 2'b00;        // Idle state

localparam START = 2'b01;        // Start bit state

localparam DATA  = 2'b10;        // Data bits state

localparam STOP  = 2'b11;        // Stop bit state

reg [1:0] state;                 // Current FSM state

// Output assignments

assign seri_out = seri_out_reg;

assign done     = done_reg;

// Main FSM and control logic

always @(posedge clk) begin

if (rst) begin

// Reset all internal registers and outputs

state        <= IDLE;

done_reg     <= 1'b0;

start        <= 1'b0;

busy         <= 1'b0;

seri_out_reg <= 1'b1;  // UART line is idle high

tx_baud_en   <= 1'b0;

bit_in       <= 3'd0;

buffer       <= 8'h00;

end else begin

case (state)

// -------------------------------

// IDLE: Wait for tx_en to start

// -------------------------------

IDLE: begin

done_reg     <= 1'b0;

seri_out_reg <= 1'b1;   // Keep line high

tx_baud_en   <= 1'b0;   // Disable baud generator

busy         <= 1'b0;

start        <= 1'b0;

if (tx_en) begin

state  <= START;   // Go to start bit

buffer <= d_in;    // Load data into buffer

bit_in <= 3'd0;    // Reset bit counter

end

end

// -------------------------------

// START: Send start bit (0)

// -------------------------------

START: begin

start        <= 1'b1;

busy         <= 1'b1;

seri_out_reg <= 1'b0;   // Start bit = 0

tx_baud_en   <= 1'b1;   // Enable baud generator

if (tx_count_baud_ready) begin

state <= DATA;      // Move to data bits on baud tick

end

end

// -------------------------------

// DATA: Send 8 data bits (LSB first)

// -------------------------------

DATA: begin

start        <= 1'b0;

busy         <= 1'b1;

seri_out_reg <= buffer[0]; // Output LSB

if (tx_count_baud_ready) begin

if (bit_in == 3'd7) begin

state  <= STOP;   // After last bit, go to stop

bit_in <= 3'd0;

end else begin

buffer <= buffer >> 1; // Shift right

bit_in <= bit_in + 1'b1;

end

end

end

// -------------------------------

// STOP: Send stop bit (1)

// -------------------------------

STOP: begin

busy         <= 1'b1;

seri_out_reg <= 1'b1;   // Stop bit = 1

if (tx_count_baud_ready) begin

state      <= IDLE; // Return to idle

tx_baud_en <= 1'b0; // Disable baud generator

busy       <= 1'b0;

done_reg   <= 1'b1; // Pulse done

end else begin

done_reg <= 1'b0;

end

end

default: state <= IDLE;

endcase

end

end

endmodule

  - Finite State Machine

  - Simulation Source

module uart_tx_tb;

// Parameters

// Baud rate parameter (17 bits required for 115200)

parameter [16:0] freq = 115200;

// Testbench Signals

reg        clk  = 1'b0;      // System clock

reg        rst  = 1'b0;      // Reset

reg [7:0]  d_in = 8'h00;     // Data input to TX

reg        tx_en = 1'b0;     // Transmit enable

// Outputs from DUT

wire seri_out;              // Serial UART output

wire done;                  // Transmission done pulse

wire start;                 // Transmission start indicator

wire busy;                  // TX busy flag

// Baud_gen interface signals for revised uart_tx

wire tx_count_baud_ready;        // Baud tick from baud_gen

wire tx_count_8x_ready_unused;  // 8x tick (unused for TX)

wire tx_baud_en;                // Enable signal from TX to baud_gen

// File I/O variables

integer file, r;

reg [7:0] stimulus_data;    // Data read from file

integer i;

// 100 MHz clock generation (10 ns period)

always #5 clk = ~clk;

// Baud generator instance

// Generates baud ticks for uart_tx

baud_gen #(

.freq(freq)

) u_baud_tx (

.clk              (clk),

.rst              (rst),

.baud_en          (tx_baud_en),          // Enabled by TX module

.count_8x_ready   (tx_count_8x_ready_unused), // Not used in TX

.count_baud_ready (tx_count_baud_ready) // Baud tick for TX

);

// DUT: Revised uart_tx (external baud_gen)

uart_tx #(

.freq(freq)

) uut (

.clk                 (clk),

.rst                 (rst),

.d_in                (d_in),

.tx_en               (tx_en),

.tx_count_baud_ready (tx_count_baud_ready),

.tx_baud_en          (tx_baud_en),

.seri_out            (seri_out),

.start               (start),

.busy                (busy),

.done                (done)

);

// Main stimulus process

initial begin

// Open stimulus file

// The file must be located in the simulation run directory

file = $fopen("stimulus.txt", "r");

if (file == 0) begin

$display("ERROR: Cannot open stimulus.txt");

$finish;

end

// Apply reset

rst   = 1'b1;

d_in = 8'h00;

tx_en = 1'b0;

#8000;                 // Hold reset for 8 us

rst   = 1'b0;

// Read and transmit 4 bytes from file

for (i = 0; i < 4; i = i + 1) begin

r = $fscanf(file, "%b\n", stimulus_data);

if (r == 1) begin

// Apply input data

d_in = stimulus_data;

// Generate a one-clock pulse on tx_en

tx_en = 1'b1;

#10;

tx_en = 1'b0;

// Wait until transmission is complete

wait (done == 1'b1);

// Display transmitted data

$display("TX Sent[%0d] = %b (0x%0h)", i, stimulus_data, stimulus_data);

// Wait before sending next byte

#15000;

end

end

// Close file and finish simulation

$fclose(file);

$display("UART TX file-based test completed.");

$finish;

end

endmodule

I  took 4 random data into a text file as each one 8 bits. I used them in the simulation and checked it.

Here they are:
00101011

11000100

11110011

01001110

  - Behavioral Simulation Waveform

Figure 9: Simulation waveform of the UART transmitter showing the serialization of input bytes (2B, C4, F3, 4E) into seri_out, along with the control signals tx_en, busy, and done, synchronized by the baud rate tick from baud_gen.

2b = 0010 1011₂ → LSB first: 1 1 0 1 0 1 0 0

c4 = 1100 0100₂ → LSB first: 0 0 1 0 0 0 1 1

f3 = 1111 0011₂ → LSB first: 1 1 0 0 1 1 1 1

4e = 0100 1110₂ → LSB first: 0 1 1 1 0 0 1 0

In the transmitter simulation, the input bytes 0x2B, 0xC4, 0xF3 and 0x4E are applied through the stimulus file. As shown in Figure X, each value is transmitted with a start bit, followed by 8 data bits in LSB-first order, and a stop bit. The busy signal stays high during the frame, while done pulses at the end of each transmission, confirming the correct operation of the UART transmitter.

  - Post-implementation Timing Waveform

Figure 10: Post-implementation timing simulation waveform of the UART transmitter, showing correct serialization of the input bytes (2B, C4, F3, 4E) on seri_out under realistic timing delays, and validating proper operation of control signals (tx_en, busy, done) with the generated baud rate ticks.

  - Utilization & Timing Summary Report

- UART Receiver

  - Design Source

module uart_rx #(

parameter [16:0] freq = 115200

)(

input  wire       clk,

input  wire       rst,

input  wire       d_in,     // Serial RX input line

input  wire       rx_en,    // Receiver enable

// Interface to external baud generator

input  wire       rx_count_8x_ready,    // 8x oversampling tick

input  wire       rx_count_baud_ready,  // Baud-rate tick

output reg        rx_baud_en,            // Enable signal to baud_gen

output reg  [7:0] d_out,   // Received parallel data

output reg        start,   // Start of reception indicator

output reg        busy,    // Receiver busy flag

output wire       done     // Reception done pulse

);

reg done_reg;

// ----------------------------------------

// Sampling and bit counters

// ----------------------------------------

reg [2:0] bit_in;         // Bit index (0 to 7)

reg [2:0] sample_cnt;     // Oversample counter (0 to 7)

reg [3:0] ones_cnt;       // (Unused here) previously for majority voting

reg [2:0] half_cnt;       // Counts 4 ticks for half-bit delay (T/2)

reg [7:0] shift_reg;      // Shift register to store received bits

// ----------------------------------------

// FSM state encoding

// ----------------------------------------

localparam IDLE  = 2'b00;

localparam START = 2'b01;

localparam DATA  = 2'b10;

localparam STOP  = 2'b11;

reg [1:0] state;

assign done = done_reg;

// ----------------------------------------

// UART RX state machine

// ----------------------------------------

always @(posedge clk) begin

if (rst) begin

// Reset all registers and go to IDLE

state      <= IDLE;

d_out      <= 8'h00;

shift_reg  <= 8'h00;

start      <= 1'b0;

busy       <= 1'b0;

done_reg   <= 1'b0;

rx_baud_en <= 1'b0;

bit_in     <= 3'd0;

sample_cnt <= 3'd0;

ones_cnt   <= 4'd0;

half_cnt   <= 3'd0;

end else begin

// Default: done is a one-cycle pulse

done_reg <= 1'b0;

case (state)

// --------------------------------

// IDLE: Wait for start bit

// --------------------------------

IDLE: begin

start      <= 1'b0;

busy       <= 1'b0;

rx_baud_en <= 1'b0;

bit_in     <= 3'd0;

sample_cnt <= 3'd0;

ones_cnt   <= 4'd0;

half_cnt   <= 3'd0;

// Detect falling edge (start bit)

if (rx_en && (d_in == 1'b0)) begin

state      <= START;

start      <= 1'b1;

busy       <= 1'b1;

rx_baud_en <= 1'b1;   // Enable baud generator

half_cnt   <= 3'd0;

sample_cnt <= 3'd0;

end

end

// --------------------------------

// START: Validate start bit at T/2

// --------------------------------

START: begin

start <= 1'b1;

busy  <= 1'b1;

if (rx_count_8x_ready) begin

if (half_cnt < 3'd3) begin

// Wait half bit time (4 ticks)

half_cnt <= half_cnt + 1'b1;

end else begin

// Sample in the middle of start bit

if (d_in == 1'b0) begin

// Valid start bit

state      <= DATA;

start      <= 1'b0;

bit_in     <= 3'd0;

sample_cnt <= 3'd0;

end else begin

// False start, return to IDLE

state      <= IDLE;

start      <= 1'b0;

busy       <= 1'b0;

rx_baud_en <= 1'b0;

end

end

end

end

// --------------------------------

// DATA: Receive 8 data bits (LSB first)

// --------------------------------

DATA: begin

busy <= 1'b1;

if (rx_count_8x_ready) begin

if (sample_cnt == 3'd7) begin

// One bit period elapsed -> sample bit

shift_reg <= { d_in, shift_reg[7:1] };

sample_cnt <= 3'd0;

if (bit_in == 3'd7) begin

state  <= STOP;   // All bits received

bit_in <= 3'd0;

end else begin

bit_in <= bit_in + 1'b1;

end

end else begin

sample_cnt <= sample_cnt + 1'b1;

end

end

end

// --------------------------------

// STOP: Wait for stop bit and finish

// --------------------------------

STOP: begin

busy <= 1'b1;

if (rx_count_baud_ready) begin

d_out      <= shift_reg;  // Output received byte

done_reg   <= 1'b1;       // Pulse done

busy       <= 1'b0;

rx_baud_en <= 1'b0;       // Disable baud generator

state      <= IDLE;

end

end

default: state <= IDLE;

endcase

end

end

endmodule

The UART receiver is designed as a finite state machine with IDLE, START, DATA and STOP states. Since the receiver was implemented before the top module, the baud rate generator is instantiated inside the receiver to provide the required baud and 8x baud timing signals. In the final top-level design, this module is later moved and shared at the top level.

In the IDLE state, the receiver waits until rx_en becomes logic high. When enabled and a falling edge is detected on the input, the FSM switches to the START state. In this state, the receiver waits for half of the baud period (T/2) to align sampling to the center of the start bit. This half-bit delay is measured by counting four ticks of the 8x baud clock.

After T/2, the FSM enters the DATA state. Here, the receiver uses the 8x baud tick as a timing reference and waits for one full bit period (eight ticks) for each data bit. At the end of each bit period, the serial input d_in is sampled once and shifted into an internal register. This process is repeated until all eight data bits are collected, as indicated by the bit counter.

After all data bits are received, the FSM switches to the STOP state. In this state, the receiver waits for one baud period corresponding to the stop bit interval and then generates a one-cycle done pulse to indicate that a byte has been received. Finally, the FSM returns to the IDLE state and waits for the next frame.

  - Finite State Machine

  - Simulation Source

module uart_rx_tb;

// ----------------------------------------

// Parameters

// ----------------------------------------

parameter [16:0] freq = 115200;              // UART baud rate (parameter to DUT)

localparam integer CLK_PERIOD     = 10;      // 100 MHz clock period in ns

localparam integer BIT_PERIOD     = 8680;    // Bit time in ns for 115200 baud

localparam integer TICK_8X_PERIOD = BIT_PERIOD/8; // 8x oversampling tick period

// ----------------------------------------

// Testbench Signals

// ----------------------------------------

reg clk  = 1'b0;         // System clock

reg rst  = 1'b0;         // Reset

reg d_in = 1'b1;         // UART RX line (idle = high)

reg rx_en = 1'b0;        // Receiver enable

// ----------------------------------------

// "baud_gen" handshake signals (generated by TB)

// These emulate the baud_gen outputs used by uart_rx

// ----------------------------------------

reg  rx_count_8x_ready   = 1'b0;  // 8x tick pulse

reg  rx_count_baud_ready = 1'b0;  // baud tick pulse (1x)

wire rx_baud_en;                 // enable from DUT to start/stop tick generation

// ----------------------------------------

// Outputs from DUT

// ----------------------------------------

wire [7:0] d_out;        // Received byte

wire done;               // Done pulse from receiver

wire start;              // Start detection indicator

wire busy;               // Receiver busy flag

// ----------------------------------------

// Comparison signals (test_data vs expected_data)

// ----------------------------------------

reg [7:0] test_data;     // Byte read from file and transmitted on d_in

reg [7:0] expected_data; // Expected received value (should match d_out)

// ----------------------------------------

// File I/O variables

// ----------------------------------------

integer file, r;

integer i;

// ----------------------------------------

// Clock Generation (100 MHz)

// ----------------------------------------

always #(CLK_PERIOD/2) clk = ~clk;

// ----------------------------------------

// Instantiate DUT (uart_rx)

// ----------------------------------------

uart_rx #(

.freq(freq)

) uut (

.clk                (clk),

.rst                (rst),

.d_in               (d_in),

.rx_en              (rx_en),

.rx_count_8x_ready   (rx_count_8x_ready),

.rx_count_baud_ready (rx_count_baud_ready),

.rx_baud_en          (rx_baud_en),

.d_out               (d_out),

.start               (start),

.busy                (busy),

.done                (done)

);

// ---------------------------------------------------------

// Generate rx_count_8x_ready pulses when rx_baud_en is high

// Also generate rx_count_baud_ready once every 8 pulses

// This emulates a baud generator used by the RX module.

// ---------------------------------------------------------

integer tick8_cnt = 0;

initial begin

rx_count_8x_ready   = 1'b0;

rx_count_baud_ready = 1'b0;

tick8_cnt           = 0;

forever begin

// Wait for the 8x tick interval

#(TICK_8X_PERIOD);

if (rx_baud_en) begin

// Create a pulse long enough so the DUT will not miss it

rx_count_8x_ready = 1'b1;

#(CLK_PERIOD);

rx_count_8x_ready = 1'b0;

// After 8x pulses, create one baud tick pulse

if (tick8_cnt == 7) begin

tick8_cnt = 0;

rx_count_baud_ready = 1'b1;

#(CLK_PERIOD);

rx_count_baud_ready = 1'b0;

end else begin

tick8_cnt = tick8_cnt + 1;

end

end else begin

// If RX disables baud generation, reset tick counters/pulses

tick8_cnt           = 0;

rx_count_8x_ready   = 1'b0;

rx_count_baud_ready = 1'b0;

end

end

end

// ----------------------------------------

// Task: Send one UART frame on d_in

// Frame format: start(0) + 8 data bits LSB-first + stop(1)

// ----------------------------------------

task send_uart_byte;

input [7:0] data;

begin

// Start bit

d_in = 1'b0;

#(BIT_PERIOD);

// Data bits (LSB first)

for (i = 0; i < 8; i = i + 1) begin

d_in = data[i];

#(BIT_PERIOD);

end

// Stop bit

d_in = 1'b1;

#(BIT_PERIOD);

end

endtask

// ----------------------------------------

// Main test sequence

// - Apply reset

// - Enable RX

// - Read bytes from stimulus.txt

// - Send each byte as a UART frame

// ----------------------------------------

initial begin

// Initial values

d_in  = 1'b1;

rst   = 1'b1;

rx_en = 1'b0;

test_data     = 8'h00;

expected_data = 8'h00;

// Hold reset briefly

#100;

rst  = 1'b0;

rx_en = 1'b1;

// Idle line before first frame (helps receiver start clean)

#(3*BIT_PERIOD);

// Open stimulus file (must exist in simulation run directory)

file = $fopen("stimulus.txt", "r");

if (file == 0) begin

$display("ERROR: Cannot open stimulus.txt");

$finish;

end

// Read until end-of-file; stimulus file expected to be binary (%b)

while (!$feof(file)) begin

r = $fscanf(file, "%b\n", test_data);

if (r == 1) begin

// Set expected value for comparison when done asserts

expected_data = test_data;

// Send UART frame

$display("Sending byte: bin=%b hex=%h", test_data, test_data);

send_uart_byte(test_data);

// Small gap between frames

#(3*BIT_PERIOD);

end

end

// Close file and finish simulation

$fclose(file);

#20000;

$finish;

end

// ----------------------------------------

// Monitor: Compare received output with expected_data

// Prints PASS/FAIL whenever done pulse occurs

// ----------------------------------------

always @(posedge clk) begin

if (done) begin

if (d_out == expected_data)

$display("PASS @%t : d_out=%h matches expected=%h", $time, d_out, expected_data);

else

$display("FAIL @%t : d_out=%h expected=%h", $time, d_out, expected_data);

end

end

endmodule

The UART receiver is designed as a finite state machine with the states IDLE, START, DATA, and STOP. Since the receiver was implemented before the top module, the baud-rate generator is instantiated inside the receiver to provide both the baud and 8x baud timing signals. In the final top-level design, this module is later moved to the top level and shared between the transmitter and receiver.

In the IDLE state, the receiver remains inactive until rx_en is asserted. When enabled and a falling edge is detected on the serial input, indicating the start bit, the FSM switches to the START state. In this state, the receiver waits for half of the baud period (T/2) in order to align the sampling instant to the center of the start bit. This half-bit delay is measured by counting four ticks of the 8x baud clock.

After T/2, the FSM enters the DATA state. Here, the receiver uses the 8x baud tick as a timing reference and waits for one full bit period (eight ticks) for each data bit. At the end of each bit period, the serial input d_in is sampled once and shifted into an internal register. This process is repeated until all eight data bits are collected, as indicated by the bit counter.

After all data bits are received, the FSM switches to the STOP state. In this state, the receiver waits for one baud period corresponding to the stop bit interval and then generates a one-cycle done pulse to indicate that a byte has been received. Finally, the FSM returns to the IDLE state and waits for the next frame.

  - Simulation Waveform

Figure 11: UART receiver simulation waveform showing start-bit detection, T/2 alignment, 8x oversampling during data reception, and the reconstructed parallel output d_out with done pulses.

In the receiver simulation, serial data frames are applied to the d_in input using the testbench while rx_en is asserted. As shown in Figure X, the receiver detects the falling edge of the start bit and waits for half a bit duration (T/2) in order to align sampling in time. Then, in the DATA state, the receiver uses the 8x baud tick as a timing reference and samples the serial input once per bit period (after 8 ticks) to reconstruct each data bit, shifting the sampled value into an internal register until 8 data bits are collected.
After the 8-bit word is reconstructed, the receiver waits one baud period corresponding to the stop bit and produces a one-cycle done pulse. The parallel output d_out matches the expected values (0x2B, 0xC4, 0xF3, and 0x4E), verifying correct operation of the UART receiver.

  - Post-implementation Timing Simulation Waveform

Figure 12: Post-implementation timing simulation waveform of the UART receiver, showing the reception of serial frames on d_in, oversampling using rx_count_8x_ready, and correct reconstruction of the parallel output d_out. The control signals start, busy, and done confirm proper frame detection and completion under realistic implementation delays.

  - Utilization & Timing Summary Reports

- TOP Module

  - Design Source

// Top-level UART module

// Integrates one baud generator, one UART transmitter, and one UART receiver

// TX and RX share the same baud_gen instance as required by the design

module uart_top #(

parameter [16:0] freq = 115200     // UART baud rate

)(

input  wire       clk,             // System clock (e.g., 100 MHz)

input  wire       rst,             // Global reset

// ----------------------------------------

// Control signals

// ----------------------------------------

input  wire       tx_en,            // Enable transmission

input  wire       rx_en,            // Enable reception

// ----------------------------------------

// Parallel TX input

// ----------------------------------------

input  wire [7:0] tx_d_in,          // Byte to be transmitted

// ----------------------------------------

// Serial lines

// ----------------------------------------

input  wire       rx_serial_in,     // Serial input from UART line

output wire       tx_serial_out,    // Serial output to UART line

// ----------------------------------------

// Parallel RX output

// ----------------------------------------

output wire [7:0] rx_d_out,         // Received byte

// ----------------------------------------

// Status signals - TX

// ----------------------------------------

output wire       tx_start,         // TX start indicator

output wire       tx_busy,          // TX busy flag

output wire       tx_done,          // TX done pulse

// ----------------------------------------

// Status signals - RX

// ----------------------------------------

output wire       rx_start,         // RX start indicator

output wire       rx_busy,          // RX busy flag

output wire       rx_done           // RX done pulse

);

// ----------------------------------------

// Internal wiring between baud_gen, TX, and RX

// ----------------------------------------

wire count_8x_ready;        // 8x oversampling tick from baud_gen

wire count_baud_ready;     // Baud-rate tick from baud_gen

wire tx_baud_en;           // TX requests baud generator enable

wire rx_baud_en;           // RX requests baud generator enable

// Enable baud_gen when either TX or RX is active

wire baud_en_top;

assign baud_en_top = tx_baud_en | rx_baud_en;

// ----------------------------------------

// Single baud generator instance

// Shared by both transmitter and receiver

// ----------------------------------------

baud_gen #(.freq(freq)) u_baud (

.clk              (clk),

.rst              (rst),

.baud_en          (baud_en_top),

.count_8x_ready   (count_8x_ready),

.count_baud_ready (count_baud_ready)

);

// ----------------------------------------

// UART Transmitter instance

// ----------------------------------------

uart_tx #(.freq(freq)) u_tx (

.clk                 (clk),

.rst                 (rst),

.d_in                (tx_d_in),

.tx_en               (tx_en),

.tx_count_baud_ready (count_baud_ready),

.tx_baud_en          (tx_baud_en),

.seri_out            (tx_serial_out),

.start               (tx_start),

.busy                (tx_busy),

.done                (tx_done)

);

// ----------------------------------------

// UART Receiver instance

// ----------------------------------------

uart_rx #(.freq(freq)) u_rx (

.clk                 (clk),

.rst                 (rst),

.d_in                (rx_serial_in),

.rx_en               (rx_en),

.rx_count_8x_ready   (count_8x_ready),

.rx_count_baud_ready (count_baud_ready),

.rx_baud_en          (rx_baud_en),

.d_out               (rx_d_out),

.start               (rx_start),

.busy                (rx_busy),

.done                (rx_done)

);

endmodule

The uart_top module integrates the UART transmitter, UART receiver, and a single shared baud rate generator into a complete UART communication system. The transmitter and receiver operate independently but share the same baud generator, which is enabled whenever either the transmitter or the receiver requests baud timing (baud_en_top = tx_baud_en | rx_baud_en). Parallel input data are serialized by the transmitter and driven onto the tx_serial_out line, while incoming serial data on rx_serial_in are decoded by the receiver and presented as parallel output on rx_d_out. Status signals indicate the start, busy, and completion of both transmission and reception, allowing external control and monitoring of UART operations.

  - Simulation Source

// Testbench for uart_top

// Performs loopback testing by connecting TX output to RX input

// Applies different phase delays and checks received data automatically

module uart_top_tb;

// ----------------------------------------

// Parameters

// ----------------------------------------

parameter [16:0] freq = 115200;      // UART baud rate

localparam integer CLK_PERIOD = 10;  // 100 MHz clock period (ns)

localparam integer BIT_PERIOD = 8680;// Bit period for 115200 baud (ns)

// ----------------------------------------

// Clock and reset

// ----------------------------------------

reg clk = 1'b0;

reg rst = 1'b0;

// ----------------------------------------

// TX / RX control and data

// ----------------------------------------

reg tx_en = 1'b0;           // Transmit enable

reg rx_en = 1'b0;           // Receive enable

reg [7:0] tx_d_in = 8'h00;  // Parallel TX input data

// ----------------------------------------

// Serial connection

// ----------------------------------------

wire tx_serial_out;         // Serial output from TX

// RX input is delayed version of TX output

wire rx_serial_in;

integer phase_ns;          // Programmable phase delay in ns

// ----------------------------------------

// Outputs / status from DUT

// ----------------------------------------

wire [7:0] rx_d_out;        // Parallel RX output

wire tx_start, tx_busy, tx_done;

wire rx_start, rx_busy, rx_done;

// ----------------------------------------

// Test vectors and expected data

// ----------------------------------------

reg [7:0] test_vec [0:3];   // Test bytes to transmit

reg [7:0] expected_data;   // Expected RX output

integer k;

// ----------------------------------------

// Clock generation (100 MHz)

// ----------------------------------------

always #(CLK_PERIOD/2) clk = ~clk;

// ----------------------------------------

// Apply phase delay between TX and RX (loopback)

// ----------------------------------------

assign #(phase_ns) rx_serial_in = tx_serial_out;

// ----------------------------------------

// DUT: uart_top instance

// ----------------------------------------

uart_top #(.freq(freq)) dut (

.clk          (clk),

.rst          (rst),

.tx_en        (tx_en),

.rx_en        (rx_en),

.tx_d_in      (tx_d_in),

.rx_serial_in (rx_serial_in),

.tx_serial_out(tx_serial_out),

.rx_d_out     (rx_d_out),

.tx_start     (tx_start),

.tx_busy      (tx_busy),

.tx_done      (tx_done),

.rx_start     (rx_start),

.rx_busy      (rx_busy),

.rx_done      (rx_done)

);

// ----------------------------------------

// Task: Send one byte through TX

// tx_en is asserted for exactly one full clock cycle

// and aligned to clock edges so uart_tx cannot miss it

// ----------------------------------------

task send_byte;

input [7:0] b;

begin

$display("send_byte called: %h at %t", b, $time);

// Drive data and assert tx_en before a rising edge

@(negedge clk);

tx_d_in <= b;

tx_en   <= 1'b1;

// Keep tx_en high for one full clock cycle

@(negedge clk);

tx_en   <= 1'b0;

$display("tx_en deasserted at %t", $time);

end

endtask

// ----------------------------------------

// Main stimulus process

// ----------------------------------------

initial begin

// Initialize test vectors

test_vec[0] = 8'h2B;

test_vec[1] = 8'hC4;

test_vec[2] = 8'hF3;

test_vec[3] = 8'h4E;

// Initial conditions

rst = 1'b1;

tx_en = 1'b0;

rx_en = 1'b0;

phase_ns = 0;

// Hold reset for some cycles

#(20*CLK_PERIOD);

rst = 1'b0;

// Enable receiver

rx_en = 1'b1;

// Small idle gap before first transmission

#(5*BIT_PERIOD);

// Loop through all test vectors

for (k = 0; k < 4; k = k + 1) begin

// Change phase between TX and RX to test robustness

phase_ns = (k * (BIT_PERIOD/4)) % BIT_PERIOD;

expected_data = test_vec[k];

$display("---- Frame %0d : sending %h, phase_ns=%0d ----",

k, expected_data, phase_ns);

// Start transmission

send_byte(expected_data);

// Wait long enough for TX and RX to complete

#(15*BIT_PERIOD);

// Compare received data with expected value

if (rx_d_out == expected_data)

$display("PASS: rx_d_out=%h matches expected=%h @%t",

rx_d_out, expected_data, $time);

else

$display("FAIL: rx_d_out=%h expected=%h @%t",

rx_d_out, expected_data, $time);

// Gap before next frame

#(3*BIT_PERIOD);

end

// Finish simulation

#20000;

$finish;

end

endmodule

The uart_top_tb testbench generates the system clock and reset, and applies four predefined test bytes (0x2B, 0xC4, 0xF3, 0x4E) to the UART transmitter through an internal test vector array. For each transmitted frame, the receiver output is compared against the expected value and pass/fail messages are reported.

  - Simulation Waveform

    - Behavioral Simulation:

Figure 13: Behavioral simulation waveform of the uart_top module showing loopback operation, where transmitted bytes (0x2B, 0xC4, 0xF3, 0x4E) are correctly received at the RX output with corresponding start, busy, and done signals.

Figure 13 shows the behavioral simulation results of the uart_top module configured in loopback mode. The input bytes are applied to the transmitter through tx_d_in, and transmission is initiated by asserting tx_en. For each test case, the transmitter generates a UART frame consisting of a start bit, eight data bits, and a stop bit, which appears on the serial line tx_serial_out. This serial stream is directly fed into the receiver input.
When a complete frame is received, the receiver reconstructs the parallel byte at rx_d_out and asserts a one-cycle rx_done signal. The waveform demonstrates that for all transmitted values (0x2B, 0xC4, 0xF3, and 0x4E), the received data matches the transmitted data, confirming correct end-to-end operation of the UART loopback system..

    - Post-implementation Timing Simulation

Figure 14: Post-implementation timing simulation of the UART top module showing correct loopback operation with gate-level delays.

Figure 14 shows the post-implementation timing simulation results of the uart_top module after place-and-route. In this simulation, the synthesized and routed netlist is used together with timing delays, providing a realistic representation of hardware behavior. Despite the presence of propagation delays, the transmitter successfully generates UART frames on the serial line tx_serial_out, and the receiver correctly reconstructs the transmitted bytes at rx_d_out. The tx_done and rx_done signals are asserted as one-cycle pulses for each frame, indicating proper completion of transmission and reception. The results demonstrate that the UART top module operates correctly not only at the behavioral level but also after implementation with actual timing effects.

  - Utilization & Timing Summary Reports

Our WHS value is 6.511ns. Therefore, 10 – 6.511= 3.489 ns. Maximum clock frequency is:
Fmax≈ 13.489ns≈286.6 MHz

  - Block Diagram

- References

[1] Nordic Semiconductor, “UART protocol,” nRF Connect SDK Fundamentals - Serial Communication (UART), Academy.nordicsemi.com, 2025. [Online]. Available: https://academy.nordicsemi.com/courses/nrf-connect-sdk-fundamentals/lessons/lesson-4-serial-communication-uart/topic/uart-protocol/ [Accessed: Dec. 28, 2025].

[2] J. Wong, “Software UART through emulated GPIO pins,” JimmyWongIoT.com, Oct. 23, 2022. [Online]. Available: https://jimmywongiot.com/2022/10/23/software-uart-through-emulated-gpio-pins/ [Accessed: Dec. 28, 2025].

[3] Seeed Studio, “UART Communication Protocol and How It Works,” Seeed Studio Blog, Sep. 8, 2022. [Online]. Available: https://www.seeedstudio.com/blog/2022/09/08/uart-communication-protocol-and-how-it-works/ [Accessed: Dec. 28, 2025].

[4] Article World, “Fundamentals of UART Communication,” ArticleWorld.com, [Online]. Available: https://aticleworld.com/fundamentals-of-uart-communication/ [Accessed: Dec. 28, 2025].

[5] Analog Devices, “UART — A Hardware Communication Protocol,” Analog Dialogue, [Online]. Available: https://www.analog.com/en/resources/analog-dialogue/articles/uart-a-hardware-communication-protocol.html [Accessed: Dec. 28, 2025].